# Project Blueprint: "Sentinel Mesh" (Mini-Lattice)

Status: Architecture Phase (Draft 1.0)

Objective: To design a decentralized, cloud-edge hybrid autonomous monitoring system that mimics the logic of "Mission Autonomy" found in platforms like Anduril’s Lattice OS.

## 1. System Philosophy

- Decentralized Intelligence: Decisions are made at the lowest level possible (Edge) and only escalated to the "Commander" (Cloud) when necessary.

- Sensor Fusion: Combining raw data from multiple sources (Video, LoRa, PIR) into a single "World State."

- Asynchronous Communication: Using gRPC and LoRa to ensure low-latency, high-reliability messaging in restricted bandwidth environments.

## 2. Hardware Architecture

### Tier 1: The Perimeter (Remote Sensors)

- Hardware: Seeed Studio XIAO ESP32-S3 with Wio-SX1262 LoRa expansion.

- Role: Low-power "tripwire" nodes.

- Function: Monitor temperature, PIR motion, or light.

- Connectivity: Raw LoRa (Sub-GHz).

- Power: LiPo battery with Solar harvesting; optimized for μA Deep Sleep.

### Tier 2: The Tactical Edge (The Gateway)

- Hardware: Raspberry Pi 5 + AI HAT+ 2 + Waveshare LoRa HAT.

- Role: The "Local Brain."

- Function: 1. Listen for LoRa packets from the Perimeter. 2. Run local Computer Vision (YOLO26) on the AI HAT+ to verify LoRa triggers. 3. Consolidate data into a "Common Operational Picture."

- Connectivity: gRPC (Outbound to Cloud), LoRa (Inbound from Perimeter).

## 3. Software & Cloud Stack

### The "Brain" (Command & Control)

- Orchestrator: Amazon Bedrock AgentCore.

- Logic: Uses a "Supervisor-Agent" model to determine responses based on a Knowledge Base (Rules of Engagement).

- Integration: AWS Lambda for executing actions (notifications, database logging, or hardware triggers).

### Data Protocols

- Perimeter-to-Edge: Custom Binary Packet Structure (Raw LoRa).

- Edge-to-Cloud: gRPC with Protocol Buffers (over TLS).

- Storage: AWS DynamoDB for the immutable event log.

## 4. Logical Workflow

- Detection: An ESP32-S3 wakes from deep sleep on a perimeter trigger and blasts a LoRa packet.

- Verification: The Pi 5 receives the packet and immediately activates its camera/AI HAT to look for a visual match.

- Escalation: The Pi 5 sends a structured Inference Event via gRPC to the Bedrock Agent.

- Decision: Bedrock analyzes the event against historical data and user preferences to determine if the "incident" is a threat or a known entity.

- Action: The system executes the appropriate response (e.g., an encrypted alert to your Pixel Watch).

## 5. Future Scalability (V2.0)

- Raspberry Pi Zero 2 W Nodes: For non-AI "Disposable" video streams.

- Digital Twin: Integrating a 3D tactical map for real-time visualization of the "Sentinel Mesh."